Test Kit Number_______________________

Keep this paper in a safe place until you receive your test results back from the lab.

Radon Testing Procedures

Prepare for the Test...

- Be sure your house has been closed (all exterior doors and windows closed) for 12 hours before you open the test kit or have your house tested.  They must remain closed the WHOLE TESTING PERIOD.  Normal activities such as opening doors for work are fine.  Run your AC/heat and existing radon mitigation system (if installed) as usual.  Do not use fireplaces during testing.

Place the Test Kit Correctly...

- Put the test kit in the lowest level of the house where you spend the most time.

• Do not test in the basement if you only go downstairs to get a jar of canned goods or to do the laundry.

• Do test in the basement if you have family members sleeping there or you have an office, hobby area or rec room where you spend a lot of time.

- Place the test kit in an occupied room.

• Do not place the test kit in a closet or other seldom-used area.

- Place the test kit away from drafts and moisture.

• Do not place the test kit near drafts, a fan or a vent to your heating and cooling system.

• Do not place the test kit near a kitchen, bathroom or garage.

- Place the test kit where it will not be disturbed:

• At least 20 inches from the floor;

• At least 4 inches from other objects;

• At least 12 inches away from an exterior wall; and

• At least 3 feet from exterior doors, windows or other openings to the outside.

Conduct the test

- Keep all doors and windows closed except for normal entry and exit.

- The building’s normal heating, ventilating and air conditioning systems as well as any existing radon mitigation systems should be allowed to operate during the test.

- If the testing period is only 2 or 3 days long, do not conduct the test during periods of severe storms or winds in excess of 30 mph.

Depending on your results, you may have retest with another short or a long term test.   It is better to have two conclusive tests before beginning any radon mitigation.

Two Rivers Public Health Department staff will be available to answer radon questions, or assist occupants with any aspect of their radon test.  Katherine may be contacted at 308-995-4778, by email kmulligan@trphd.org or stop at our office which is located at 701 4th Ave. Suite 1, Holdrege, NE.